# Practical Networking Troubleshooting Guide

L1 / L2 / L3 • Switching • Routing • Multicast • Transport • Day-to-Day NOC Scenarios

The best way to make all of this click is to stop treating BGP, ARP, STP, trunks, PIM, OSPF, EtherChannel, etc. as separate things to memorize. They are different pieces of the same forwarding path.

I’d organize your troubleshooting mentally like this:

Power/physical → Ethernet switching → IP/ARP → routing → application/service

That maps roughly to L1 → L2 → L3 → service.

This guide uses Cisco-style commands because that matches most of the environment discussed. Exact syntax varies somewhat by IOS/IOS-XE/NX-OS/IOS-XR.

# 1. The master troubleshooting model

Suppose you get: STTN router down, NYL user has no connectivity, or Te0/0/0/7.2006 down. Don’t immediately think “BGP.” Ask: How far through the path can traffic get?

User
 |
AP
 |
Access Switch
 |
Trunk / EtherChannel
 |
Distribution/Core
 |
LAN interface/SVI
 |
Router
 |
WAN interface
 |
Carrier handoff / Ciena / NID
 |
Carrier network
 |
Remote PE/router
 |
BGP/OSPF/etc.
 |
Destination

Each command answers a question about one part of this path.

# 2. L1 — Is there actually a physical connection?

Layer 1 doesn’t care about IP addresses, VLANs, BGP, ARP or routing. It cares about power, cable/fiber, optics, signal, interface/link state, speed/duplex, and physical errors.

show interfaces status
show interfaces Gi1/0/10
show interfaces Te0/0/0/7
show controllers
show inventory
show logging

For fiber platforms you may also use commands for transceiver/optical levels.

# 3. Understanding up/up, down/down, up/down

This is extremely useful.

GigabitEthernet0/0/0 is up, line protocol is up

up/up: physical connection exists and the protocol is operational. Generally good.

down/down: think L1 first — cable, carrier handoff, NID/Ciena, optic, remote equipment, shutdown on the other side, or physical circuit failure.

Gi0/0/0 is administratively down

Administratively down/down means someone or the configuration shut it with shutdown. Usually not a carrier outage.

up/down means physical carrier is detected, but the protocol is not functioning correctly. Investigate encapsulation, keepalives, serial/controller problems, PPP/HDLC, remote configuration, or another protocol-specific issue. This distinction becomes especially important with serial/T1 tickets.

# 4. The interface command you should know extremely well

show interfaces Gi0/0/0

Don’t just look at up, line protocol is up. Look through the counters.

5 minute input rate 2000000 bits/sec
5 minute output rate 8000000 bits/sec

100000 packets input
20 input errors
20 CRC
20 frame

200000 packets output
0 output errors

What are you trying to prove? CRC errors mean frames arrived but failed their checksum. Think bad cable, dirty/damaged fiber, optic, carrier handoff, electrical interference, NID/ONT, or another physical-layer issue.

clear counters Gi0/1

If you clear counters and then see CRC values rise quickly — 0, 15, 42, 98 — you have strong evidence of an active physical problem, not merely historical errors.

# 5. Input errors vs CRC vs drops

Don’t treat all interface counters as equivalent.

CRC / frame errors: usually a physical-quality concern.

Input errors: a broader receive-side counter that can include several problems.

Output drops: a different story.

0 CRC
0 input errors
250000 output drops

5 minute output rate 48 Mbps

On a 50-Mbps circuit, that points more toward congestion, queue exhaustion, or utilization than bad fiber. This is the conceptual difference between bandwidth tickets and CRC/circuit tickets.

# 6. Your uptime/BGP method is good — but don’t make it absolute

show version
show ip bgp summary

If router uptime is 47 days but the BGP neighbor has been up only 37 minutes, the router did not reboot; the BGP session re-established 37 minutes ago.

show logging | include BGP
show logging | include Gi0/0/0

Gi0/0/0 changed state to down
Gi0/0/0 changed state to up
%BGP-5-ADJCHANGE: neighbor ... Down
%BGP-5-ADJCHANGE: neighbor ... Up

The sequence matters: WAN down → BGP down → WAN up → BGP up. That is strong evidence BGP failed because underlying connectivity failed, not that BGP itself caused the outage.

# 7. Matching router uptime and BGP uptime doesn’t prove power

Router uptime: 3 hours 17 minutes
BGP uptime:    3 hours 15 minutes

That strongly suggests the router restarted and BGP formed shortly afterward. Possible reasons include site power loss, manual reboot, hardware crash, reload after maintenance, software failure, or power-supply issues.

show version
show logging
show environment

A safer initial conclusion is: “Router appears to have reloaded approximately 3 hours ago; BGP re-established shortly afterward.” Then power verification can establish the cause.

# 8. L2 — where switches, VLANs, trunks, STP and EtherChannel live

Imagine the router is fine, WAN is fine, and BGP is fine, but NYL users on floor 3 cannot connect. WAN troubleshooting becomes a waste of time. You are probably in L2/LAN territory.

Important concepts: MAC addresses, VLANs, access ports, trunks, STP, PortFast, EtherChannel, port-channel, and LACP.

# 9. Access port

interface Gi1/0/15
 switchport mode access
 switchport access vlan 300
 spanning-tree portfast

PC
 |
Gi1/0/15
 |
VLAN 300

The PC generally does not need to know VLAN 300 exists. The switch associates its traffic with VLAN 300.

show interfaces Gi1/0/15 switchport
show vlan brief
show interfaces status
show mac address-table interface Gi1/0/15

# 10. What show vlan brief proves

show vlan brief

VLAN Name             Status
1    default          active
100  USERS            active
200  VOICE            active
300  WIFI             active

You are answering: Does this VLAN exist locally and is it active? If VLAN 300 should be used but does not appear, that is immediately suspicious. But this does not prove end-to-end VLAN connectivity. One command rarely proves everything.

# 11. Trunk ports

A trunk is how multiple VLANs cross one physical or logical link.

VLAN100 ---- cable
VLAN200 ---- cable
VLAN300 ---- cable

Without a trunk, you would conceptually need separate links. Instead:

Switch A
   |
   | VLAN 100
   | VLAN 200
   | VLAN 300
   |
 trunk
   |
Switch B

interface Gi1/0/48
 switchport mode trunk
 switchport trunk allowed vlan 100,200,300

show interfaces trunk

Port      Mode   Encapsulation Status
Gi1/0/48  on     802.1q        trunking

Vlans allowed on trunk
100,200,300

# 12. Classic trunk troubleshooting scenario

NYL users in VLAN 300 suddenly lose connectivity. The switch itself is reachable and the uplink is up.

show interfaces trunk

Vlans allowed on trunk
100,200

VLAN 300 is missing.

PC -> access port VLAN300
                  |
                  X
             uplink trunk

The physical link can be perfectly healthy while users remain down. L1 is working; L2 is broken.

# 13. MAC address table

This is one of your best LAN troubleshooting tools.

show mac address-table
show mac address-table interface Gi1/0/15

300   aaaa.bbbb.cccc   DYNAMIC   Gi1/0/15

That tells you the switch learned that MAC address from Gi1/0/15 in VLAN 300. It is evidence frames from the device are reaching the switch.

show interfaces Gi1/0/15
show interfaces Gi1/0/15 switchport
show mac address-table interface Gi1/0/15

Physical link?
     ↓
Correct VLAN?
     ↓
Learning MAC?
     ↓
Then move toward ARP/IP.

# 14. ARP — extremely important for your work

ARP answers: What MAC address owns this IPv4 address on my directly connected network?

interface Gi0/0/1
 ip address 10.50.1.1 255.255.255.0

If a user is 10.50.1.25, the router needs a destination MAC to send the Ethernet frame.

Who has 10.50.1.25?
10.50.1.25 is at aaaa.bbbb.cccc

10.50.1.25 -> aaaa.bbbb.cccc

show ip arp
show ip arp 10.50.1.25

# 15. ARP + MAC table together is powerful

show ip arp 10.50.1.25

10.50.1.25    aaaa.bbbb.cccc

show mac address-table address aaaa.bbbb.cccc

300  aaaa.bbbb.cccc  DYNAMIC  Gi1/0/22

IP
10.50.1.25
   ↓ ARP
MAC
aaaa.bbbb.cccc
   ↓ MAC table
Switch port
Gi1/0/22

You just traced IP → MAC → switch port. This is one of the most useful L2/L3 correlations you can make.

# 16. When ARP is useful

ping 10.50.1.1
!!!!!

ping 10.50.1.25
.....

show ip arp 10.50.1.25

No ARP entry can point toward endpoint disconnected, wrong VLAN, switch path problem, wrong IP/subnet, or broken L2 path.

If an ARP entry exists, L2 reachability has progressed much farther. Maybe the endpoint firewall blocks ICMP, there is an endpoint/application issue, or there is a return-path problem. ARP helps narrow the fault domain.

# 17. STP — why it exists

Ethernet switches need redundancy, but redundant Layer-2 paths can create loops.

SW1 -------- SW2
 |            |
 |            |
 +---- SW3 ---+

A broadcast such as an ARP request could circulate repeatedly. STP exists to create a loop-free logical topology while retaining redundant physical links.

# 18. What STP actually does

STP elects a root bridge and determines which links should forward and which redundant paths should be blocked/discarding.

show spanning-tree
show spanning-tree vlan 300

You will see roles such as Root, Designated, and Alternate, and states such as Forwarding and Blocking/Discarding depending on the STP version/platform.

Do not immediately think blocked = broken. A blocked/discarding redundant port can be exactly what STP is supposed to do.

# 19. STP troubleshooting

show spanning-tree vlan 300
show spanning-tree interface Gi1/0/48 detail
show logging | include SPANTREE

Look for topology changes, root bridge changes, ports moving states, unexpected blocked paths, inconsistent ports, BPDU Guard, and loop-related events.

A key question: Did STP cause the outage, or did STP react to another link going down? Sequence matters.

# 20. PortFast

For an endpoint such as PC ---- Gi1/0/10, there normally is not another switch creating a loop.

spanning-tree portfast

PortFast lets an edge port enter forwarding quickly.

interface Gi1/0/10
 switchport mode access
 spanning-tree portfast

Use it for edge/end-device ports according to the network design. Do not blindly put PortFast on switch-to-switch links.

# 21. BPDU Guard

BPDU Guard goes naturally with PortFast. If a user plugs an unexpected switch into a PortFast edge port, BPDUs can arrive where they should not.

BPDU Guard can protect the network by err-disabling the port.

%SPANTREE-2-BLOCK_BPDUGUARD

show interfaces status err-disabled
show logging | include BPDU

This is why an access port can be physically connected yet unusable.

# 22. EtherChannel and Port-channel

Suppose two switches need more bandwidth and redundancy. Multiple physical links can be bundled into one logical interface.

Gi1/0/47 \
          \
           Po1 ===== SW2
          /
Gi1/0/48 /

The logical interface is Port-channel1. The physical members could be Gi1/0/47 and Gi1/0/48.

# 23. Why EtherChannel?

Three major benefits:

Bandwidth — multiple links contribute capacity.

Redundancy — one member can fail while the port-channel remains operational.

STP simplicity — STP sees the logical port-channel rather than several independent parallel links.

# 24. LACP

EtherChannel can use LACP. Typical modes are active and passive; at least one side generally needs to be active.

interface range Gi1/0/47-48
 channel-group 1 mode active

interface Port-channel1
 switchport mode trunk

show etherchannel summary

Group  Port-channel  Protocol  Ports
1      Po1(SU)       LACP      Gi1/0/47(P) Gi1/0/48(P)

The letters matter. P typically indicates the member is bundled properly. If one member is suspended or not bundled, investigate.

# 25. EtherChannel failure scenario

Ticket: Po1 down.

show etherchannel summary
show interfaces Po1
show interfaces Gi1/0/47
show interfaces Gi1/0/48
show logging

If Gi1/0/47 is down, Gi1/0/48 is up, and Po1 remains up, you have degraded redundancy but not necessarily complete outage.

If both physical members are down and Po1 is down, the entire logical uplink is gone. This distinction matters when assessing customer impact.

# 26. L3 — now IP routing begins

At Layer 3, routers answer: Where should this packet go?

show ip route

C 10.10.10.0/24 is directly connected
L 10.10.10.1/32 is directly connected
O 10.20.0.0/16 via 10.1.1.2
B 172.16.0.0/16 via 10.2.2.2
S 0.0.0.0/0 via 192.0.2.1

C = connected
L = local
O = OSPF
B = BGP
S = static

Those route codes tell you where the routes came from.

# 27. Routing table vs BGP table

show ip bgp

This asks: What routes does BGP know?

show ip route

This asks: What routes did the router actually install for forwarding? A route can exist in BGP without becoming the route used by the router. Routing-protocol knowledge is not the same thing as the forwarding decision.

# 28. OSPF

OSPF is an IGP — Interior Gateway Protocol. Think: How do routers inside one routing domain learn the internal topology?

R1 ----- R2 ----- R3

OSPF routers establish neighbor relationships, exchange link-state information, build a topology database, and calculate best paths using SPF.

show ip ospf neighbor
show ip ospf interface
show ip route ospf
show ip ospf database

# 29. OSPF neighbor states

DOWN
INIT
2-WAY
EXSTART
EXCHANGE
LOADING
FULL

For a normal established adjacency, you are generally looking for FULL.

Neighbor ID     State       Address
10.1.1.2        FULL/DR     192.168.1.2

If stuck in EXSTART or EXCHANGE, think about things such as MTU mismatch. If no neighbor exists at all, investigate interface down, IP/subnet mismatch, OSPF not enabled correctly, area mismatch, authentication, ACL/network issue, or hello/dead timer mismatch. Exact diagnosis depends on topology.

# 30. BGP

BGP has a different purpose. Think: Exchange routes between routing domains or policy boundaries.

In your environment you often care less about Internet-scale theory and more about whether the WAN BGP neighbor is established, whether it flapped, whether you are receiving routes, and whether you are advertising what you should.

show ip bgp summary

Neighbor       State/PfxRcd
10.1.1.1       247

If established, that final column generally shows the number of prefixes rather than the word Established. Idle, Active, or Connect are unhealthy states that require investigation.

# 31. Don’t troubleshoot BGP before proving reachability

If the BGP neighbor is 10.1.1.1, first ask: Can I reach 10.1.1.1?

show interfaces <WAN>
show ip interface brief
show ip route 10.1.1.1
ping 10.1.1.1
ping 10.1.1.1 source <WAN-IP>

L1
 ↓
L2/transport
 ↓
IP reachability
 ↓
TCP/179
 ↓
BGP

If the underlying IP path does not work, staring at BGP configuration is not your first priority.

# 32. BGP troubleshooting ladder

1. Router reachable?
2. Router uptime?
3. WAN interface up?
4. WAN interface errors?
5. Neighbor IP reachable?
6. Correct route to neighbor?
7. Logs show interface flap?
8. Logs show BGP reason?
9. Configuration correct?
10. Carrier/remote peer issue?

show version
show ip interface brief
show interfaces <WAN>
show ip route <neighbor>
ping <neighbor> source <source>
show ip bgp summary
show logging | include BGP
show logging | include <WAN>
show running-config | section router bgp

That is much more systematic than starting with show run | sec bgp.

# 33. OSPF and BGP can coexist

BGP
Customer R1 -------- Provider
    |
    | OSPF
    |
Internal R2
    |
Internal R3

OSPF may provide internal reachability while BGP provides WAN/external routes. Neither makes the other redundant.

OSPF + BGP
EIGRP + BGP
static + BGP
connected + BGP

The exact combination depends on network design.

# 34. PIM — don’t think of it as another BGP

PIM is for multicast routing.

Unicast:
Source ----> one destination

Multicast:
              Receiver A
             /
Source ---- Router ---- Receiver B
             \
              Receiver C

Rather than sending separate streams across every part of the network, multicast can replicate traffic where necessary. PIM helps routers build multicast distribution paths.

show ip pim neighbor

You are asking: Which directly connected multicast routers am I forming PIM relationships with?

# 35. Why PIM can be on core routers but not necessarily edge routers

If multicast is required only across certain parts of the infrastructure, those interfaces participate in PIM. An edge router that only needs ordinary unicast WAN routing might use BGP but have no reason to participate in PIM.

A core router carrying multicast may have both BGP/IGP and PIM.

BGP/OSPF determine unicast reachability. PIM uses that underlying reachability to construct multicast paths. They are related, but they are not doing the same job.

# 36. PIM troubleshooting

show ip pim neighbor
show ip pim interface
show ip mroute
show ip igmp groups

IGMP = hosts tell the network which multicast groups they want
PIM = routers figure out how multicast moves between routers
mroute = multicast forwarding state

If a PIM neighbor disappears, check the PIM neighbor table and then the underlying interface and logs. Ask whether PIM broke independently or whether its interface/path flapped. Use the same troubleshooting philosophy as BGP.

# 37. Subinterfaces — your Te0/0/0/7.2006 type alerts

Te0/0/0/7 is physical. Te0/0/0/7.2006 is a logical subinterface.

interface TenGigabitEthernet0/0/0/7.2006
 encapsulation dot1q 2006
 ip address ...

Physical Te0/0/0/7
        |
        +-- .100
        +-- .200
        +-- .2006

One physical interface can carry multiple logical L3 connections using VLAN tags. If .2006 alerts, do not automatically assume the physical Te0/0/0/7 failed. Check both.

# 38. Why a subinterface may disappear

If an alert references Te0/0/0/7.2006 but show ip interface brief no longer shows it, possibilities include planned configuration change, service removal, migration, stale monitoring, or intentional decommissioning.

This is where the operational process matters: Is there a VTM/change associated with it? If yes, the alert may simply be an expected consequence of a planned change.

# 39. Gi, Te, Se, Po — learn to read the ticket before touching anything

Gi = Gigabit Ethernet. It could be LAN, WAN handoff, switch access port, or switch uplink. You need topology/context.

Te = Ten-Gigabit Ethernet. Often higher-capacity uplinks/core connections, but do not assume solely from interface type.

Se = Serial. In your environment this can point you toward T1, controller, channel, carrier transport, framing, and clocking.

Po = Port-channel. Then ask which physical interfaces are members.

show etherchannel summary

# 40. Serial/T1 troubleshooting

Serial/T1 troubleshooting differs from normal Ethernet. For something like Serial0/1/0:1 you may need to investigate both the serial interface and its controller.

show interfaces serial 0/1/0:1
show controllers
show controllers t1
show logging

Look for loss of signal, loss of frame, alarms, slips, framing errors, line-code violations, controller state, and carrier state.

# 41. Controller down vs serial interface down

Carrier T1
   |
Physical T1 controller
   |
Channel/time slot
   |
Serial interface
   |
IP/routing

If the controller itself is down, the serial interface above it cannot function.

Controller down
       ↓
Serial down
       ↓
BGP/OSPF/etc down

If you are troubleshooting only BGP at that point, you are several layers too high.

# 42. SONET — what that means

“Sonnet” here is SONET — Synchronous Optical Networking, an older carrier optical transport technology.

Your router
    |
interface
    |
carrier access
    |
SONET transport
    |
carrier network

It is not BGP and it is not Ethernet switching. It is part of the transport infrastructure carrying service underneath the routed connection.

SONET-related alarms can involve loss of signal, loss of frame, path alarms, section/line alarms, and transport failure. When you see a SONET/controller problem, move downward and ask whether the carrier transport underneath the IP service is failing.

# 43. Transport vs CPE — the question you deal with constantly

One of the most important skills in your job is determining whether the fault is customer/CPE, carrier/transport, or configuration/change.

Customer/CPE?
     or
Carrier/transport?
     or
Configuration/change?

Think in evidence rather than assumptions.

# 44. Strong CPE/power indicators

Examples include router unreachable, OOB unreachable, multiple local devices down, router recently rebooted, uptime aligning with outage/restoration, or the site confirming power loss.

Site power
   ↓
Router down
Switch down
APs down

Carrier investigation is not necessarily the first move.

# 45. Strong transport indicators

Router uptime: 73 days
WAN interface: down
LAN: up
router reachable through OOB
carrier handoff alarmed
BGP down exactly when WAN dropped

Router alive
     |
LAN alive
     |
WAN X
     |
Carrier

That strongly points toward transport.

Another strong example is router alive, WAN up, and CRC errors rapidly incrementing. This could still involve CPE cabling/optic, so isolate the demarcation, but you have established an active physical-path problem.

# 46. Ciena/NID/OOB can dramatically change the diagnosis

If the router is unreachable but a managed carrier/NID/OOB device remains alive, something upstream still has power/connectivity. This does not automatically prove the carrier circuit is healthy, but it narrows the possibilities.

What is reachable?
What isn’t?
Where is the boundary between those two?

That is fault isolation.

# 47. Your Tennessee workflow

Because STTN is primarily router/WAN responsibility for your team, your scope is simpler.

Reachability
OOB
Power
Router uptime
WAN interface
Controller
Transport
BGP/routing
Logs
Known NERS
Known VTM/change

You generally do not need to spend 30 minutes investigating an access switch or AP when LAN is outside your responsibility. That boundary matters operationally.

# 48. NERS vs VTM — conceptual distinction

Treat these as operational workflow references rather than networking protocols.

NERS: a reference associated with an outage/event means there is already a broader network/event record tracking the condition. Correlate the ticket to that known event instead of treating every downstream symptom as an unrelated failure.

VTM: change-related. If you discover an interface removed, routing modified, or service migrated and it appears intentional, ask whether it was associated with an approved change. If a change is required rather than repair, the LCM/SM workflow comes into play for creating the VTM.

The networking skill is recognizing fault vs intentional configuration change.

# 49. NCR — understand where they fit in the OSI troubleshooting process

The NCR technician is essentially remote physical hands. You remain the network engineer.

Does the device have power?
What LEDs are showing?
Is the cable connected?
Which port?
Can you reseat it?
Can you replace a cable?
Can you console into it?
Can you move the console connection?
Can you read the model/serial?

Then you use the console session for show version, show ip interface brief, show run, show interfaces, show logging, and configuration work. The CE is not deciding the VLAN architecture or changing BGP independently; you use the CE to extend your hands to the physical site.

# 50. When console access is valuable

If a router is unreachable, you do not yet know whether it is dead, boot-looping, in ROMMON, has a bad config, has interfaces down, has routing broken, or only has the management path broken.

If NCR arrives, the power LED is green, and console shows:

rommon 1 >

that is a huge difference. The router is powered but failed to boot normally. Console access converts a vague “router down” symptom into a much more precise device/boot-state finding.

# 51. Wi-Fi troubleshooting should follow the same layered method

If an NYL user says Wi-Fi keeps disconnecting, do not start by rebooting the AP.

Client
 ↓
Association
 ↓
Authentication
 ↓
VLAN
 ↓
DHCP
 ↓
Gateway
 ↓
Routing/WAN

Ask: Can the client see the SSID? Can it associate? Did authentication succeed? Did it receive an IP? Is it in the correct subnet? Is the gateway correct? Can it ping the gateway? Can it reach outside? Is only one client affected, one AP, one floor, or the whole site?

That fault-domain thinking is more valuable than memorizing one WLC command.

# 52. DHCP fits between L2 and usable L3 connectivity

169.254.x.x

If a user connects to Wi-Fi but gets an APIPA address, it is a major clue. The client associated sufficiently to attempt networking but did not obtain a normal DHCP lease.

DHCP
VLAN
trunk
relay/helper
scope
switch path

Investigate those before BGP. A healthy WAN does not help a client that never received an IP.

# 53. VRF — another concept from your daily work

A VRF is essentially a separate routing table.

Router
 |
+-- VRF CUSTOMER_A
|     routing table A
|
+-- VRF MIS_INTERNET
|     routing table B
|
+-- VRF OIR
      routing table C

The same physical router can maintain logically separated routing domains.

ping vrf MIS_INTERNET <gateway> source <LAN-interface>

You are telling the router to test connectivity using this routing table and originate the traffic from this source. A normal ping could test the wrong routing context and mislead you.

# 54. Why source pings matter

ping 8.8.8.8

A successful generic ping does not necessarily prove customer traffic works. The router might source that ping from an interface with completely different routing.

ping <destination> source <customer-LAN-interface>
ping vrf <VRF> <destination> source <source>

This tests a path closer to what customer traffic experiences. You are not merely proving the router has connectivity; you are trying to prove that customer routing context has connectivity.

# 55. show ip interface brief — your first snapshot

show ip interface brief

This quickly answers what interfaces exist, what IPs they have, and whether they are up/up, down/down, or administratively down.

Interface       IP-Address      Status     Protocol
Gi0/0/0         192.0.2.2       up         up
Gi0/0/1         10.20.1.1       up         up
Gi0/0/2         unassigned      down       down

You immediately see WAN up, LAN up, and an unused interface down. Do not treat every down/down as a problem. Context matters.

# 56. show run interface

show running-config interface Gi0/0/0

Once an interface looks suspicious, this asks: What is this interface supposed to be doing?

Look for description, IP address, VRF, encapsulation, shutdown/no shutdown, speed, duplex, channel-group, switchport mode, allowed VLANs, PIM, and OSPF-related configuration.

description *** VERIZON WAN ***

description *** CUSTOMER LAN ***

The description can immediately tell you which direction you are troubleshooting.

# 57. Logs — don’t just grep; reconstruct the timeline

show logging | include BGP
show logging | include <interface>

10:13:02 Gi0/0/0 down
10:13:02 line protocol down
10:13:03 BGP neighbor down
10:17:45 Gi0/0/0 up
10:17:47 line protocol up
10:18:03 BGP neighbor up

Physical/WAN event
      ↓
IP connectivity lost
      ↓
BGP adjacency lost
      ↓
physical service restored
      ↓
BGP automatically re-established

That is much stronger than writing “BGP flapped.” The BGP flap was a symptom.

# 58. Another log pattern

No interface flap
BGP neighbor Down - Hold Timer Expired
BGP neighbor Up

This is different. The WAN may have remained physically up while packets stopped reaching the peer. Possible areas include packet loss, carrier impairment, congestion, remote peer, or control-plane issues. Interface error/utilization counters become especially important.

# 59. Router-down ticket — your practical workflow

ROUTER DOWN

Can I ping it?
     |
     NO
     ↓
OOB/device around it reachable?
     |
Check known outage/change
     |
Power verification
     |
NCR if physical access needed
     |
Console

If the router becomes reachable:

show version
show ip interface brief
show interfaces <WAN>
show ip bgp summary
show logging

Power/reload?
Transport?
Routing?
Change?
CPE hardware?

# 60. WAN/interface-down ticket

Example: Gi0/0/0 down.

show ip interface brief
show interfaces Gi0/0/0
show run interface Gi0/0/0
show logging | include Gi0/0/0

Ask: Admin down? Physical down? Errors? Recent flap? Description says carrier? Neighbor protocol affected? Known change?

If carrier-facing, use show ip bgp summary and appropriate source ping to the peer as applicable.

# 61. Switch-down ticket

Your thought process should change.

SWITCH DOWN

Ask: Ping? Does upstream see its uplink? MAC/ARP? Power? Other switches at site affected? Stack? Uplink? Port-channel? STP?

show interfaces status
show interfaces <uplink>
show etherchannel summary
show spanning-tree
show logging

If the upstream port is down/down and the remote switch is unreachable, you are moving toward switch power, cabling, optic, or hardware. That is where NCR may become appropriate.

# 62. AP-down ticket

An AP depends on more than just Wi-Fi.

AP
 |
Ethernet
 |
PoE
 |
Switch
 |
VLAN
 |
IP/DHCP
 |
WLC/controller/cloud

AP down can actually mean AP hardware, switch port, PoE, VLAN, DHCP, uplink, or controller reachability.

show interfaces status
show interfaces <AP-port>
show power inline
show mac address-table interface <AP-port>
show logging

Before replacing an AP, prove whether it is even receiving power and Ethernet connectivity.

# 63. Switch stack problems

show switch
show switch stack-ports

A ticket saying “switch down” might actually mean the stack is reachable but one member is missing. That is not the same as an entire stack being unreachable. Determine the actual failure domain.

# 64. CDP/LLDP — something important you didn’t mention

show cdp neighbors
show cdp neighbors detail
show lldp neighbors
show lldp neighbors detail

These answer: What network device is physically/logically connected to this port?

SW1 Gi1/0/48
        |
        ?

Device ID: SW2
Local Interface: Gi1/0/48
Port ID: Gi1/0/48

This is invaluable when documentation is incomplete.

# 65. Route tracing

traceroute <destination>

A ping tells you destination reachable/not reachable. Traceroute helps answer: How far does traffic get?

1 10.1.1.1
2 172.16.1.1
3 *
4 *

This gives a possible boundary. It is not absolute proof hop 3 is broken because devices can suppress traceroute responses, but it is useful evidence.

# 66. ACLs — another missing piece

You can have interface up, ARP good, routing good, and BGP good, yet traffic still fails because of filtering.

ip access-group
access-list

Routing says:
"send packet this way"

ACL says:
"you’re not allowed."

Those are separate decisions.

# 67. Default gateway concept

IP:      10.10.10.25
Mask:    255.255.255.0
Gateway: 10.10.10.1

For destination 10.10.10.50 on the same subnet, the PC ARPs directly for .50. For destination 8.8.8.8 on a different subnet, the PC ARPs for gateway 10.10.10.1 and sends the frame there.

If the gateway is unreachable, off-subnet communication dies even if the PC and switch are otherwise functioning. This is where ARP, VLAN, and routing concepts meet.

# 68. A complete NYL user connectivity example

Customer says: One workstation cannot reach the network. Do not start at Verizon. Work upward.

Step 1 — physical

show interfaces Gi1/0/22

Is it up/up?

Step 2 — VLAN

show interfaces Gi1/0/22 switchport

Correct access VLAN?

Step 3 — MAC

show mac address-table interface Gi1/0/22

Is the client MAC learned?

Step 4 — ARP

show ip arp <client-IP>

Does IP resolve to MAC?

Step 5 — gateway: can client reach gateway?

Step 6 — routing

show ip route <destination>

Step 7 — upstream: test with appropriate source/VRF.

Notice how much evidence you gathered before touching BGP.

# 69. A complete Tennessee router-down example

Alert: STTN router unreachable.

Check 1: Ping router. Fails.

Check 2: Check OOB/associated equipment.

Check 3: Check known NERS/VTM. No known event.

Check 4: Power verification. Site says power exists — but “building has power” is not identical to the router being powered and operational.

Check 5: NCR if appropriate. CE arrives, router power LED is on, and console is available.

show version
show ip interface brief
show interfaces <WAN>
show controllers
show ip bgp summary
show logging

Suppose router uptime is 43 days, Serial0/1/0:1 is down/down, T1 controller is down, and BGP is down.

Controller/transport
      ↓
Serial interface
      ↓
IP connectivity
      ↓
BGP

That is a transport investigation, not “BGP troubleshooting.”

# 70. Another Tennessee example — BGP down but interface up

Router uptime: 82 days

Gi0/0/0:
up/up

BGP:
Active

ping <peer> source Gi0/0/0

If that fails, check ARP, route to peer, interface counters, and logs.

show arp
show ip route <peer>
show interfaces Gi0/0/0
show logging

Suppose the interface is up/up with 0 CRC and 0 errors, but the peer is unreachable. Carrier transport can still be broken because Ethernet carrier at your handoff can remain up while connectivity deeper inside the provider network is broken.

up/up does not prove an end-to-end circuit is healthy. It proves only certain lower-layer conditions at that interface.

# 71. The same principle with BGP

BGP Established does not prove every destination works. BGP can be established while a specific route is missing, the wrong route is selected, a route is filtered, the VRF is wrong, the application is failing, the LAN is broken, or multicast has a problem.

Every command has a scope of what it proves. That mindset is critical.

# 72. What common commands actually prove

| Command | Question you are answering |
| --- | --- |
| show version | Did device reboot? What platform/software? |
| show ip int brief | Which L3 interfaces are operational? |
| show interfaces X | What is happening physically/operationally on X? |
| show run interface X | What is X configured to do? |
| show logging | What happened and in what order? |
| show ip arp | Which IPv4-to-MAC mappings does this device know? |
| show mac address-table | Where did the switch learn MAC addresses? |
| show vlan brief | Which VLANs exist locally? |
| show interfaces trunk | Which links trunk and which VLANs cross them? |
| show spanning-tree | What L2 topology has STP selected? |
| show etherchannel summary | Are bundle members actually participating? |
| show cdp neighbors detail | What network device is connected where? |
| show ip route | Which L3 paths are installed? |
| show ip bgp summary | Are BGP sessions established? |
| show ip bgp | What does BGP know? |
| show ip ospf neighbor | Are OSPF adjacencies healthy? |
| show ip pim neighbor | Are directly connected PIM routers present? |
| show ip mroute | What multicast forwarding state exists? |
| show controllers | What does the underlying hardware/controller see? |
| ping | Can packets reach and return from this target? |
| traceroute | Approximately how far does the path progress? |

# 73. The troubleshooting hierarchy I want you using at work

Instead of memorizing 100 ticket types, use a consistent decision process every time:

ALERT
                      |
                      v
             Is device reachable?
               /             \
             YES              NO
              |                |
              v                v
        What is affected?    Power/OOB?
              |                |
      +-------+-------+        |
      |       |       |        v
     WAN     LAN    Protocol   CPE/transport
      |       |       |
      v       v       v
 Interface  VLAN     Underlying
 errors?    MAC      reachability
      |      ARP       first
      |      STP       |
      |      trunk     v
      |      Po       BGP/OSPF/PIM
      |
      v
Carrier handoff
      |
      v
Transport

Was there a change?
Was there a known event?
What do the logs say?
What changed immediately before the symptom?

# 74. Your biggest next-level improvement

You already know many commands from repetition. The gap is not simply learning more commands; it is knowing exactly what conclusion you are and are not allowed to draw from an output.

show ip bgp summary showing Established means the BGP control-plane adjacency is established. It does not mean “the network is fine.”

Gi0/0/0 up/up means the interface currently has operational lower-layer state. It does not mean the carrier’s entire end-to-end circuit is good.

show ip arp showing an endpoint means the router has learned an IPv4-to-MAC mapping. It does not mean the application works.

That distinction separates someone who knows Cisco commands from someone who can isolate faults.

# 75. The four questions to ask before every command

Before typing something, mentally ask:

1. What layer am I testing? L1 / L2 / L3 / routing protocol / service.

2. What question am I asking? For show interfaces Gi0/0/0: Is my WAN interface physically healthy, and what are its counters?

3. What result do I expect if healthy? Up/up, no significant errors, reasonable utilization.

4. If it is unhealthy, what layer do I investigate next?

That is actual troubleshooting.

# Compact command ladder for day-to-day tickets

You do not run every command every time. Move downward or upward depending on the symptom.

DEVICE / BASELINE
-----------------
show version
show clock
show ip interface brief
show logging


PHYSICAL / INTERFACE
--------------------
show interfaces <interface>
show run interface <interface>
show controllers
show inventory


SWITCHING / L2
--------------
show interfaces status
show interfaces <port> switchport
show vlan brief
show interfaces trunk
show mac address-table
show spanning-tree
show spanning-tree vlan <vlan>
show etherchannel summary
show cdp neighbors detail
show lldp neighbors detail


L3
--
show ip arp
show ip route
show ip route <destination>
ping <destination>
ping <destination> source <source>
traceroute <destination>


VRF
---
show ip route vrf <VRF>
ping vrf <VRF> <destination> source <source>


BGP
---
show ip bgp summary
show ip bgp
show ip route <BGP-prefix>
show logging | include BGP


OSPF
----
show ip ospf neighbor
show ip ospf interface
show ip route ospf
show ip ospf database


MULTICAST
---------
show ip pim neighbor
show ip pim interface
show ip igmp groups
show ip mroute


SERIAL / T1
-----------
show interfaces serial <interface>
show controllers
show controllers t1
show logging

The important part is not running this as one giant command dump. It is knowing which branch applies to the alert.

For a Se0/x/x:x down ticket, controller/serial/transport commands move near the top. For Gi1/0/20 down on an NYL access switch, VLAN/MAC/STP/PoE may matter. For a Te... alert on a Tennessee core, first determine whether it is a physical interface, subinterface, PIM-facing/core link, or service interface and whether a known change explains it.

The operational experience you already have makes this easier to learn: router power events, BGP flaps, Ciena/NID problems, T1 controller failures, CRC accumulation, missing subinterfaces, PIM alerts, bandwidth saturation, AP/WLC problems, Port-channel links, and customer LAN issues. The useful next step is to connect each real ticket to what happened at L1 → L2 → L3 and why the commands exposed it.